UNPLUGGED 2025 — Round 2

Product Requirements Document

KiCad PCB Design — Line Follower Car Control System

Team VorteX  |  DJSCE  |  24-Hour Hardware Hackathon

Time Constraint: 4 Hours  |  Tool: KiCad

# 1. Problem Statement Summary

Design and develop a high-efficiency schematic and PCB layout in KiCad for a Line Follower Car's central control system. The PCB must fit within the given chassis dimensions, interface all 11 specified components correctly, maintain signal integrity, and pass ERC/DRC checks — all within 4 hours.

Optional (Brownie Points): Design an LTspice simulation of a polarity-independent 12V–50V input system with regulated +12V output and polarity indicator logic.

# 2. PCB Mechanical Constraints

The PCB must conform to the chassis reference shape. These are hard constraints — the board outline must match.

| Dimension | Value (mm) | Notes |
| --- | --- | --- |
| Overall Width (top) | 125.18 | Top horizontal span |
| Overall Width (bottom) | 167.05 | Bottom horizontal span |
| Total Height | 181.19 | Full vertical height |
| Top-left notch height | 13.71 | Top recessed area |
| Right step width | 20.86 | Right side step inset |
| Right step height | 11.89 | Right step drop |
| Left mid-section height | 65.32 | Left vertical section |
| Angled corner cutoff | 61.71 | Diagonal cut on lower-left |
| Bottom-left section height | 46.04 | Bottom left extension |

⚡ The board outline (Edge.Cuts layer in KiCad) must exactly replicate the L/stepped shape shown in the reference. Draw this first before placing any components.

# 3. Component List & Placement Strategy

All 11 components must be present in the schematic with correct footprints. Below is the BOM with placement guidance.

| # | Component | Footprint Type | Power Rail | Placement Zone |
| --- | --- | --- | --- | --- |
| 1 | ESP32 | Module (38-pin) | 3.3V | Center — main controller area |
| 2 | ESP32-CAM | Module (16-pin) | 3.3V / 5V | Near ESP32, top region |
| 3 | TB6612 Motor Driver | IC (24-SSOP or module) | 5V logic / VM=12V | Near motors, bottom area |
| 4 | 12V DC Gear Motor (x2) | Through-hole connector | 12V | Bottom edge, motor connectors |
| 5 | NEO-6M GPS | Module (6-pin header) | 3.3V | Edge, away from motor noise |
| 6 | IR Sensor Module (x3+) | 3-pin header per sensor | 5V | Bottom edge, forward-facing |
| 7 | Solar Panel | 2-pin connector | Input to TP4056 | Edge connector — top |
| 8 | 18650 Battery Holder | Through-hole 2-pin | 3.7V (Li-Ion) | Bottom-left large zone |
| 9 | TP4056 Charger | Module (6-pin) or IC | In: solar ¦ Out: battery | Near battery holder |
| 10 | Buck Module LM2596 | Module (5-pin) or IC | In: 12V ¦ Out: 5V/3.3V | Center-right power section |
| 11 | 0.96" OLED (I2C) | 4-pin header (SDA,SCL,VCC,GND) | 3.3V | Top-center, accessible edge |

💡 Group components by function: Power zone (battery, TP4056, LM2596, buck), Control zone (ESP32, OLED, GPS), Motor zone (TB6612, motor connectors), Sensor zone (IR sensors, ESP32-CAM).

# 4. Power Distribution Architecture

## 4.1 Power Flow Diagram (Text)

Solar Panel → TP4056 (charger) → 18650 Battery (3.7V)

18650 Battery → LM2596 Buck #1 → 5V rail → TB6612 logic, IR Sensors, OLED, ESP32-CAM

18650 Battery → LM2596 Buck #2 (or LDO 3.3V) → 3.3V rail → ESP32, NEO-6M GPS

External 12V input (or boosted) → TB6612 VM pin → Motor power

## 4.2 Power Rail Summary

| Rail | Voltage | Source | Consumers |
| --- | --- | --- | --- |
| VBAT | 3.7V (Li) | 18650 battery | LM2596 inputs |
| +5V | 5V regulated | LM2596 Buck | TB6612 logic, IR sensors, OLED, CAM |
| +3V3 | 3.3V regulated | LM2596 / LDO | ESP32, NEO-6M GPS |
| VM | 12V motor | External / boost | TB6612 motor power pins only |
| GND | 0V common | All returns tied | All components |

⚡ Add 100nF decoupling capacitors at every IC power pin and 10µF bulk capacitors near each power rail. This is critical for passing ERC.

# 5. Signal Interface Map (ESP32 Pin Assignments)

These are recommended pin assignments. Confirm against ESP32 datasheet for any ADC/PWM constraints.

| Interface | Protocol | ESP32 Pins | Connected To | Notes |
| --- | --- | --- | --- | --- |
| Motor Control | PWM + GPIO | GPIO25, 26, 27, 14, 12, 13 | TB6612 (IN1,IN2,PWMA,IN3,IN4,PWMB) | Use LEDC PWM channels |
| IR Sensors | Digital IN | GPIO34, 35, 32 (ADC capable) | IR OUT pins | Use INPUT_PULLUP; avoid GPIO 6-11 |
| GPS | UART | GPIO16 (RX2), GPIO17 (TX2) | NEO-6M TX→GPIO16, RX→GPIO17 | Use Serial2; 9600 baud |
| OLED Display | I2C | GPIO21 (SDA), GPIO22 (SCL) | OLED SDA/SCL | Add 4.7kΩ pull-ups on both lines |
| ESP32-CAM | UART or WiFi | GPIO1 (TX), GPIO3 (RX) | CAM module serial | Or WiFi stream via same WiFi chip |
| Standby TB6612 | GPIO | GPIO33 | TB6612 STBY pin | Pull HIGH to enable motor driver |

💡 Add a 10kΩ pull-up from STBY pin to 5V on the PCB so TB6612 is always enabled unless explicitly disabled by GPIO.

# 6. KiCad Step-by-Step Workflow (4-Hour Plan)

Follow this exact sequence. Do NOT start routing before the schematic is fully complete and ERC passes.

## ⏱ Phase 1 — Schematic (0:00 – 1:30)

- Open KiCad → New Project → Name it 'VorteX_LinFollower'

- Open Eeschema (Schematic Editor)

- Add power symbols first: VCC_3V3, VCC_5V, VCC_12V, GND — from 'power' library

- Place ESP32 symbol — search 'ESP32' in symbol library (use ESP32-WROOM-32 or generic module)

- Place all other components from the library. For modules (TP4056, LM2596), use generic module footprints with headers

- Wire all connections using 'W' key. Label nets: +3V3, +5V, VMOT, GND, SDA, SCL, GPS_TX, GPS_RX, IR1, IR2, IR3, PWMA, PWMB, IN1-IN4

- Add power flags (PWR_FLAG) to all power nets to avoid ERC errors

- Add decoupling capacitors (100nF) next to each IC

- Run ERC (Tools → Electrical Rules Checker) — fix all errors before moving on

⚡ ERC errors for 'pin unconnected' on module pins you don't use — add a small 'x' No-Connect marker (press Q) on those pins.

## ⏱ Phase 2 — Footprint Assignment (1:30 – 2:00)

- Tools → Assign Footprints (or press 'A' per component)

- Assign footprints as listed in the table below

- Run Tools → Update PCB from Schematic to push changes

| Component | KiCad Footprint Path | Notes |
| --- | --- | --- |
| ESP32 | Module:ESP32-WROOM-32 | Standard 38-pin module |
| ESP32-CAM | Module:ESP32-CAM (or generic 16-pin header) | Use 2x8 pin header if no specific footprint |
| TB6612 | Package_SO:SSOP-24 or Module | Use module version with header pins |
| Motors | Connector_PinHeader_2.54mm:PinHeader_1x02 | 2-pin screw terminal per motor |
| NEO-6M GPS | Module header 1x6 2.54mm | PinHeader_1x06_P2.54mm |
| IR Sensor | Module header 1x3 2.54mm (per sensor) | PinHeader_1x03_P2.54mm |
| Solar Panel | Connector 1x2 screw terminal | TerminalBlock_2.54mm or similar |
| Battery Holder | BatteryHolder_18650_2x | Keystone 2x18650 or 1x |
| TP4056 | Module 1x6 header or IC DFN-8 | Use module header version |
| LM2596 Buck | Module 5-pin header or TO-263-5 | Use module header version |
| OLED 0.96" | Module header 1x4 2.54mm | GND, VCC, SCL, SDA pinout |

## ⏱ Phase 3 — PCB Layout (2:00 – 3:30)

- Open PCB Editor (Pcbnew)

- FIRST: Draw board outline on Edge.Cuts layer using the exact dimensions from Section 2

- Set design rules: Min trace width = 0.25mm (signal), 0.8mm (power), 1.5mm (motor). Clearance = 0.2mm minimum

- Arrange components by zone: Power zone (left side), Control zone (center), Sensor zone (bottom edge), Motor zone (bottom-right)

- Place connectors (motors, IR sensors, GPS, OLED) on board edges for easy access

- Route power traces first (thick traces): GND, +5V, +3V3, VMOT

- Add copper pour (ground plane) on both F.Cu and B.Cu — Edit → Zones → Add Filled Zone → GND

- Route signal traces last (thin traces)

- Press 'B' to fill all copper pours

- Run DRC (Inspect → Design Rules Checker) — fix all errors

⚡ Use the Ratsnest (thin lines showing unconnected nets) as your guide. Each ratsnest line = a missing trace. Zero ratsnest lines = fully routed.

💡 If time is short, use the Auto-Router (Freerouting plugin) for signal traces, but manually route all power traces.

## ⏱ Phase 4 — Verification & Export (3:30 – 4:00)

- Run ERC (Schematic) — 0 errors required

- Run DRC (PCB) — 0 errors required (warnings acceptable)

- Generate 3D view: View → 3D Viewer (screenshot this for submission)

- File → Plot → Gerber files (for all copper layers, Edge.Cuts, silkscreen)

- File → Fabrication Outputs → BOM

- Screenshot: complete PCB layout, F.Cu layer, B.Cu layer, 3D render, schematic

- Push all KiCad project files to GitHub repository

# 7. PCB Routing & Signal Integrity Rules

| Net Type | Min Width (mm) | Recommended (mm) | Notes |
| --- | --- | --- | --- |
| Signal (GPIO, I2C, UART) | 0.25 | 0.3 | Short routes, avoid parallel to motor lines |
| 5V / 3.3V power | 0.5 | 0.8 | Star topology from buck converter |
| Motor power (VMOT) | 1.0 | 1.5–2.0 | Thick traces, short paths to TB6612 VM |
| GND pour | N/A | Copper fill | Both layers, stitched with vias |
| I2C (SDA/SCL) | 0.25 | 0.3 | Keep traces < 30cm, add 4.7kΩ pull-ups |
| UART (GPS) | 0.25 | 0.3 | Keep away from motor traces |

⚡ Never route UART/I2C traces parallel to motor PWM traces. Separate with GND pour between them.

# 8. Deliverables Checklist

## 8.1 Mandatory PCB Deliverables

- KiCad project files (.kicad_sch, .kicad_pcb, .kicad_pro)

- Schematic screenshot (high resolution, full view)

- Complete PCB layout screenshot (all layers visible)

- Front copper layer (F.Cu) screenshot — green traces on black

- Back copper layer (B.Cu) screenshot

- 3D rendered view of the PCB (View → 3D Viewer → screenshot)

- ERC report — must show 0 errors

- DRC report — must show 0 errors

- Bill of Materials (BOM) — exported from KiCad

## 8.2 Optional CAD Deliverables (Bonus)

- SolidWorks / Fusion 360 / FreeCAD model of chassis enclosure

- PCB mounted inside CAD chassis model (import KiCad STEP file)

- Rendered images showing structural, thermal, and accessibility features

## 8.3 Brownie Points — LTspice Simulation

- LTspice schematic: Full-bridge rectifier (4x diodes) for polarity independence

- Flyback / isolated DC-DC converter stage for +12V regulated output

- Voltage comparator circuit for polarity detection (0V = normal, 5V = reversed)

- Optocoupler for input-output electrical isolation

- Simulation screenshots: Input waveform, output 12V (both polarities), logic signal

## 8.4 GitHub Repository Structure

- README.md — project overview, setup instructions, block diagram

- /kicad/ — all KiCad schematic and PCB files

- /gerbers/ — exported Gerber manufacturing files

- /bom/ — Bill of Materials CSV

- /screenshots/ — all required screenshots

- /cad/ — optional CAD files

- /ltspice/ — optional simulation files

# 9. Brownie Points — LTspice Circuit Design

Design a polarity-independent power interface system with the following specifications:

## 9.1 System Requirements

| Parameter | Specification | Implementation |
| --- | --- | --- |
| Input voltage range | 12V to 50V DC | Full-bridge diode rectifier at input |
| Input polarity | Unknown / both | Full-bridge ensures always +ve at output |
| Output voltage | +12V regulated | Isolated DC-DC converter (flyback/push-pull) |
| Polarity indicator | 0V = normal, 5V = reversed | Voltage comparator + logic gate |
| Isolation | Input-to-output isolated | Optocoupler (PC817 or similar) |
| Validation | LTspice simulation waveforms | Show both polarity conditions |

## 9.2 Circuit Blocks to Build in LTspice

- Full-bridge rectifier: 4x 1N4007 diodes in H-bridge config — input across AC nodes, output is always +ve

- Buck-boost or flyback converter: LT3748 or generic transformer model for isolation + 12V regulation

- Polarity detection: Comparator (LT1011) comparing raw input +ve to GND. If reversed, output flips to 5V

- Isolation: PC817 optocoupler between input sense and output logic

- Test both cases: V1=+24V and V1=-24V — show that output 12V remains stable and logic signal flips

# 10. 4-Hour Execution Timeline

| Time | Duration | Task | Owner |
| --- | --- | --- | --- |
| 0:00 – 0:15 | 15 min | Setup KiCad project, set design rules, read chassis dims | All |
| 0:15 – 1:00 | 45 min | Draw full schematic — all components, power nets, labels | 2 people |
| 1:00 – 1:20 | 20 min | Run ERC, fix all errors, assign footprints | 1 person |
| 1:20 – 1:40 | 20 min | Push schematic to PCB, draw board outline (Edge.Cuts) | 1 person |
| 1:40 – 2:40 | 60 min | Component placement by zone, power trace routing | 2 people |
| 2:40 – 3:20 | 40 min | Signal trace routing, copper pour fill, silkscreen labels | 2 people |
| 3:20 – 3:40 | 20 min | DRC, fix errors, 3D view screenshots | 1 person |
| 3:40 – 4:00 | 20 min | GitHub commit, screenshots, BOM export, submission | All |
| Parallel track | Ongoing | LTspice simulation (Brownie Points) — run alongside | 1 person dedicated |

# 11. Judging Criteria — Score Maximization

| Criteria | Weight | How to Excel |
| --- | --- | --- |
| Schematic completeness & correctness | High | All 11 components, correct nets, ERC = 0 errors |
| PCB dimensional accuracy (chassis fit) | High | Edge.Cuts matches all 9 dimensions exactly |
| Component placement & routing quality | High | Logical zones, clean traces, no DRC errors |
| Power distribution & signal integrity | High | Correct rails, decoupling caps, ground pour |
| ERC pass (0 errors) | Mandatory | Required — fix all before submission |
| DRC pass (0 errors) | Mandatory | Required — fix all before submission |
| 3D view quality | Medium | Correct 3D footprints, realistic component placement |
| GitHub repo documentation | Medium | Clear README, all files organized, BOM included |
| CAD model (optional) | Bonus | Fusion 360 / SolidWorks with PCB integrated |
| LTspice simulation (Brownie) | Bonus | All 4 blocks working, both polarity waveforms shown |

# 12. KiCad Quick Reference (Shortcuts)

## Schematic Editor (Eeschema)

| Key | Action |
| --- | --- |
| A | Add symbol (component) |
| W | Draw wire |
| P | Add power symbol (VCC, GND) |
| L | Add net label |
| Q | Add no-connect marker |
| E | Edit component properties |
| R | Rotate component |
| G | Grab (move with connections) |

## PCB Editor (Pcbnew)

| Key | Action |
| --- | --- |
| X | Route trace |
| B | Fill all copper pours (ground plane) |
| E | Edit selected item properties |
| R | Rotate component |
| U | Select connected trace |
| Ctrl+B | Remove all copper fills |
| V | Switch to via / change layer while routing |
| T | Select Edge.Cuts layer to draw board outline |
| Inspect → DRC | Run Design Rule Check |

Team VorteX — UNPLUGGED 2025 | Dwarkadas J. Sanghvi College of Engineering | This PRD is for internal planning use only.