PRODUCT REQUIREMENTS DOCUMENT

Autonomous Safari Navigation System

with Real-Time Tracking and Object Detection

UNPLUGGED — A 24-Hour Hardware Hackathon  |  Round III

| Organizer | DJSCE × IETE-ISF & DJS MicroMinds VLSI Club |
| --- | --- |
| Document Version | 1.0 — April 2026 |
| Track Type | Hardware + Embedded AI + Web Integration |
| Duration | 24 Hours |

# 1. Project Overview

## 1.1 Problem Statement

Design and build an Autonomous Safari Navigation System — a self-driven robotic vehicle that navigates a predefined line-follower track resembling a safari environment. The vehicle must use IR sensors for autonomous line-following, detect animal figurines placed along the track using an onboard vision system (ESP32-CAM), transmit real-time positional data to a web dashboard, and finalize the run upon detecting an RFID checkpoint — after which the system auto-generates a run summary report.

## 1.2 Objectives

- Build a functional line-following robot using IR sensors and a motor driver on an ESP32 platform.

- Integrate an ESP32-CAM module for real-time onboard object detection (animal classification).

- Transmit live vehicle position and detection events to a web-based visualization dashboard.

- Use an RFID module to trigger end-of-run events and generate an automated summary report.

- Integrate ancillary sensors (AQI, GPS) for environmental context and enhanced localization.

- Optionally support mobile app control for manual override navigation.

## 1.3 Success Criteria

- Vehicle completes the full track autonomously without derailing more than 2 times.

- At least 3 distinct animal figurines are detected and logged with correct classification.

- Web dashboard updates in real-time (latency < 2 seconds) showing trajectory and detections.

- RFID checkpoint triggers end-of-run, and a summary report is generated within 10 seconds.

- All hardware and software components work in an integrated, cohesive prototype.

# 2. System Architecture

## 2.1 High-Level Architecture

The system is divided into four primary subsystems that communicate with each other in a coordinated pipeline:

| Subsystem | Components | Role | Communication |
| --- | --- | --- | --- |
| Locomotion & Sensing | ESP32 #1, IR Sensor, Motor Driver TB6612FNG, Motors (x2), Caster Wheel | Line following, steering control, motor PWM | GPIO / PWM |
| Vision & Detection | ESP32-CAM Module, FTDI USB-to-Serial | Capture frames, run object detection, publish detection events | Serial / HTTP POST / WebSocket |
| Event & Checkpoint | RFID Module, Switch, ESP32 #2 | Detect start/end checkpoints, trigger report generation | SPI / GPIO |
| Power & Charging | 18650 Li-ion (x2), TP4056, Buck Module, Solar Panel, 1.5V Battery | Regulated power delivery to all modules | Hardwired |
| Data & Visualization | Web Server (Node.js / Flask), WebSocket, Browser Dashboard | Receive telemetry, render live track map, log events, export report | Wi-Fi / HTTP / WebSocket |

## 2.2 Data Flow

IR sensors → ESP32 #1 (PID motor control) → Motors

ESP32-CAM → detect animal → HTTP POST → Web Server → WebSocket → Dashboard

RFID reader → ESP32 #2 → HTTP POST end-of-run event → Server → Report generation

GPS Module → ESP32 #2 → HTTP POST coordinates → Server → Map overlay

AQI Sensor → ESP32 #2 → HTTP POST → Server → Dashboard environmental widget

# 3. Functional Requirements

## 3.1 Autonomous Navigation

| ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-N1 | Vehicle shall follow a black line on white surface using minimum 3 IR sensors in a reflectance-based array configuration. | Critical | Physical demo on track |
| FR-N2 | Vehicle shall implement PID control for smooth line tracking without excessive oscillation. | Critical | Observe physical smoothness |
| FR-N3 | Vehicle shall handle curves, S-bends, crossovers, and figure-8 intersections as present on the official track layout. | Critical | Track completion test |
| FR-N4 | System shall support optional Bluetooth/Wi-Fi mobile app override for manual steering. | Medium | App control demo |
| FR-N5 | Vehicle shall recover autonomously after a brief line-loss event within 2 seconds. | High | Deliberate line-loss test |

## 3.2 Object Detection (Vision System)

| ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-V1 | ESP32-CAM shall capture JPEG frames continuously while the vehicle is in motion. | Critical | Camera stream check |
| FR-V2 | System shall run an optimized object detection model (TFLite / FOMO / custom CNN) on the ESP32-CAM to classify animal figurines placed beside the track. | Critical | Detection accuracy test |
| FR-V3 | System shall achieve a minimum detection accuracy of 70% across the predefined set of animal categories. | High | Accuracy metric on test set |
| FR-V4 | Each detected animal shall be assigned a timestamp and approximate track position before being transmitted to the server. | High | Dashboard log verification |
| FR-V5 | The detection pipeline shall run at a minimum of 3 FPS to avoid missing objects at operational vehicle speed. | High | FPS measurement |
| FR-V6 | System shall avoid duplicate logging of the same object within a 3-second window. | Medium | Duplicate suppression test |

## 3.3 RFID Checkpoint System

| ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-R1 | System shall use an RFID reader module (RC522 or equivalent) to detect a pre-programmed RFID tag embedded at the finish checkpoint. | Critical | Tag read test |
| FR-R2 | Upon RFID detection, the vehicle shall stop autonomously and transmit an end-of-run signal to the web server. | Critical | End-of-run demo |
| FR-R3 | System shall record the exact timestamp of RFID trigger to compute total elapsed run time. | High | Timestamp verification |

## 3.4 Real-Time Telemetry & Localization

| ID | Requirement | Priority | Verification |
| --- | --- | --- | --- |
| FR-T1 | ESP32 shall transmit positional telemetry (GPS coordinates or IR-derived track position) to the web server at a minimum 2 Hz rate. | Critical | Dashboard update rate test |
| FR-T2 | Telemetry payload shall include: timestamp, position (x, y or lat/lon), heading, speed estimate, and current sensor state. | High | Payload schema check |
| FR-T3 | GPS module shall supplement IR-based position with absolute geospatial coordinates when satellite fix is available. | Medium | GPS fix verification |
| FR-T4 | AQI sensor readings shall be sampled every 5 seconds and included in telemetry stream. | Low | Dashboard widget check |

# 4. Non-Functional Requirements

| Category | Requirement | Target Metric |
| --- | --- | --- |
| Performance | Vehicle must complete the full track within a reasonable time without stalling for > 5 seconds. | < 5 min total run |
| Latency | Dashboard must reflect new telemetry and detection events within 2 seconds of occurrence. | ≤ 2 seconds |
| Reliability | System shall remain operational for the entire 24-hour hackathon period with battery hot-swap or charging support. | Zero unplanned shutdowns |
| Power Stability | Buck module must deliver stable regulated voltage (3.3V / 5V) to all peripherals under load. | Ripple < 50mV |
| Connectivity | All Wi-Fi communication shall occur over a local hotspot / LAN to avoid internet dependency. | 100% local network |
| Scalability | Web server architecture shall support adding more sensor data types without refactoring core pipeline. | Modular API design |
| Usability | Dashboard shall be viewable on any modern browser without plugin installation. | Chrome/Firefox/Safari |

# 5. Hardware Component Roles & Specifications

| # | Component | Role in System | Key Specifications / Notes |
| --- | --- | --- | --- |
| 1 | ESP32 Dev Board (x2) | Board #1: Line-following logic, motor control, IR sensor reading. Board #2: RFID, GPS, AQI, Wi-Fi telemetry hub. | Dual-core 240 MHz, Wi-Fi + BT, 34 GPIO pins, 4MB flash |
| 2 | ESP32-CAM Module | Onboard vision: captures frames, runs inference for animal classification, publishes detection events via HTTP. | OV2640 camera, 4MB PSRAM, supports JPEG streaming |
| 3 | IR Sensor Module | Array of reflectance sensors to detect black line on white floor. Feeds data to PID controller. | Minimum 3 sensors recommended; digital + analog output |
| 4 | Motor Driver TB6612FNG | Drives two DC motors independently with PWM speed control and direction switching. | 1.2A per channel, 3V–13.5V motor voltage, STBY pin for sleep |
| 5 | Motors (x2) + Caster Wheel | Differential drive locomotion. Caster wheel provides stable 3-point contact for balance. | N20 or TT motor; speed matched pair recommended |
| 6 | RFID Module (RC522) | Reads RFID tags embedded in track at start/finish checkpoint to trigger event logging. | SPI interface, 13.56 MHz, 3.3V logic |
| 7 | GPS Module | Provides absolute geospatial coordinates for map overlay on the dashboard. | UART interface, NMEA sentences, NEO-6M or similar |
| 8 | AQI Sensor Module | Measures particulate matter / gas concentration; displayed as environmental context widget on dashboard. | MQ-series or PMS5003 variant; analog or UART |
| 9 | 18650 Li-ion (x2) + Holder | Primary power source for the entire vehicle. Buck module steps down to required rail voltages. | 3.7V nominal, ~3000mAh each; series = 7.4V supply |
| 10 | TP4056 | Li-ion charging IC with overcharge/discharge protection. Enables solar + USB charging. | 1A max charge current, USB-C or micro-USB input |
| 11 | Solar Panel (Small) | Trickle charges battery during daylight operation, extending run time for outdoor scenarios. | 5V / 1W minimum recommended |
| 12 | Buck Module | Converts battery voltage to stable 5V (for ESP32 / driver) and 3.3V (for sensors/RFID/GPS). | LM2596 or MP2307; efficiency > 85% |
| 13 | FTDI USB-to-Serial | Programs ESP32-CAM (which lacks native USB) and enables serial debug logging. | CH340 or FT232RL; 3.3V logic level |

# 6. Software Requirements

## 6.1 Firmware (ESP32 #1 — Navigation Controller)

- Language: C++ via Arduino IDE / PlatformIO

- Libraries: analogRead, Wire, PID_v1 or custom PID implementation

- Implements: IR sensor calibration routine, PID loop for motor control, Wi-Fi connection + HTTP POST for telemetry, Bluetooth serial (optional) for mobile app override

## 6.2 Firmware (ESP32-CAM — Vision Node)

- Language: C++ via Arduino IDE / PlatformIO

- Model: TensorFlow Lite for Microcontrollers (TFLM) or Edge Impulse FOMO model

- Implements: JPEG capture at 320×240 or 160×120, inference on captured frame, HTTP POST to web server with detection label + confidence + timestamp, suppression window to avoid duplicate detections

- Model Training: Dataset of animal figurines (lion, elephant, giraffe, rhino, etc.) — minimum 100 images per class, trained using Edge Impulse or Google Teachable Machine, exported as .tflite INT8 quantized model

## 6.3 Firmware (ESP32 #2 — Telemetry Hub)

- Handles: RFID SPI communication, GPS UART parsing (TinyGPS++ library), AQI sensor ADC reading, aggregation and periodic HTTP POST of all telemetry to web server

## 6.4 Web Server

- Recommended Stack: Node.js + Express + Socket.io (or Python Flask + Flask-SocketIO)

- Endpoints:

- POST /telemetry — receives positional data from ESP32 #2

- POST /detection — receives animal detection events from ESP32-CAM

- POST /rfid — receives checkpoint trigger + run completion flag

- GET /report — returns JSON / PDF summary of completed run

- WebSocket /live — pushes real-time events to connected dashboard clients

## 6.5 Web Dashboard

- Framework: Vanilla JS + Canvas API, or React.js / Vue.js

- Features:

- Live track map: vehicle trajectory rendered as an animated path over the predefined track SVG/image

- Detection panel: scrolling log of detected animals with icon, label, confidence, time, and position marker on map

- Timer widget: elapsed time counter (starts on run begin, stops on RFID trigger)

- AQI widget: live air quality reading

- Status bar: Wi-Fi signal, battery level estimate, vehicle speed

- Report modal: auto-triggered on run completion, shows total time + animal list + GPS path

# 7. Key Challenges & Mitigation Strategies

| Challenge | Risk | Mitigation Strategy |
| --- | --- | --- |
| PID Line-Following Tuning | Oscillation or track loss at curves and crossovers | Pre-tune Kp/Ki/Kd values on a straight track before full run. Use weighted centroid error calculation. |
| ESP32-CAM Inference Speed | Model too slow; animals missed at operating speed | Use FOMO or MobileNet V1 with INT8 quantization. Reduce input resolution to 96×96. |
| Wi-Fi Latency / Packet Loss | Dashboard lags or drops events during fast motion | Use UDP for high-frequency position updates; TCP for detection events. Run local AP on a dedicated router. |
| Power Budget | Battery drains before RFID endpoint is reached | Calculate total current draw pre-build. Use two cells in series with buck regulators. Keep ESP32-CAM inference duty-cycled. |
| Track Intersection (Figure-8) | Vehicle confused at crossover, takes wrong branch | Use center IR sensor dominance logic. Implement debounce timer to prevent direction flip at intersections. |
| Dataset Quality / Lighting | Detection model fails under hackathon lighting conditions | Capture training images under similar lighting. Augment with brightness / contrast variations. |
| System Integration Time | 24-hour limit — insufficient time for full integration | Modularize firmware. Test each subsystem independently first. Prioritize: navigation → RFID → detection → dashboard. |

# 8. Recommended Development Milestones (24-Hour Plan)

| Phase | Time Window | Deliverable | Owner |
| --- | --- | --- | --- |
| Phase 0 | H+0 to H+1 | Hardware assembly: chassis, motor wiring, IR array mounting, power rail setup and verification. | All members |
| Phase 1 | H+1 to H+4 | Firmware: Basic line-following on ESP32 #1. Test straight line and gentle curves. | Firmware Team |
| Phase 2 | H+4 to H+7 | PID tuning on full track layout. Handle S-bends, crossovers, figure-8 section. | Firmware Team |
| Phase 3 | H+4 to H+8 | Train detection model on Edge Impulse. Export TFLite. Flash to ESP32-CAM. Verify inference output. | AI/Vision Team |
| Phase 4 | H+7 to H+10 | Web server setup. REST endpoints for telemetry, detection, RFID. WebSocket live push. | Software Team |
| Phase 5 | H+10 to H+14 | Web dashboard: live track map, detection log, timer, AQI widget, report modal. | Software Team |
| Phase 6 | H+14 to H+18 | Integration: Connect all ESP32 boards to server. Validate telemetry pipeline end-to-end. | Full Team |
| Phase 7 | H+18 to H+21 | Full system test run. RFID checkpoint trigger. Report generation. Bug fixes. | Full Team |
| Phase 8 | H+21 to H+24 | Polish, presentation prep, final demo rehearsal. Document edge cases and known limitations. | Full Team |

# 9. Auto-Generated Run Report Specification

Upon RFID checkpoint trigger, the system shall automatically compile and display/export a summary report containing:

| Report Field | Description | Source |
| --- | --- | --- |
| Run Start Time | Timestamp when vehicle first moved (or switch pressed) | ESP32 #2 |
| Run End Time | Timestamp when RFID tag was detected at finish | RFID Module |
| Total Elapsed Time | End - Start in seconds and MM:SS format | Server computed |
| Total Animals Detected | Count of unique detection events logged | Detection log |
| Animal Detection Log | Table: Animal name ¦ Confidence ¦ Time ¦ Track Position | ESP32-CAM events |
| Vehicle Path | Recorded trajectory coordinates overlaid on track map (PNG export) | GPS / telemetry |
| Average AQI | Mean air quality index reading across run duration | AQI Sensor |
| Export Format | PDF download + on-screen dashboard modal + JSON API response | Web server |

# 10. Evaluation Criteria Alignment

| Criterion | What Judges Will Evaluate | Max Points | Weight |
| --- | --- | --- | --- |
| Autonomous Navigation | Vehicle completes track without human intervention. Clean cornering, no stalling. | 25 | 25% |
| Object Detection Accuracy | Correct classification of animal figurines. Detection confidence and uniqueness. | 20 | 20% |
| Real-Time Dashboard | Live trajectory map, detection markers, timer, AQI all functional and updating. | 20 | 20% |
| RFID & Report Generation | Checkpoint triggers end-of-run. Report auto-generated with all required fields. | 15 | 15% |
| Hardware-Software Integration | Seamless end-to-end operation. All modules communicating reliably. | 10 | 10% |
| Innovation & Code Quality | Unique features, clean architecture, edge case handling, modularity. | 10 | 10% |

## 10.1 Bonus Points Opportunities

- Mobile app integration for real-time control override

- Solar charging circuit functional during demo

- Multi-class animal detection with > 85% accuracy

- PDF report download functional within 10 seconds of RFID trigger

- GPS path accurately overlaid on map within 5 meter accuracy

# 11. Assumptions, Constraints & Out of Scope

## 11.1 Assumptions

- The track is the official UNPLUGGED Round 3 layout (S-bends, figure-8, oval sections) printed on white surface with black line.

- Animal figurines will be placed statically beside the track (not moving).

- Wi-Fi access point is provided or self-hosted via mobile hotspot.

- Ambient lighting is sufficient for ESP32-CAM to capture usable frames without additional illumination.

- All provided components are functional and undamaged at the start of the hackathon.

## 11.2 Constraints

- Hardware is limited to the provided component list — no additional purchases allowed mid-hackathon.

- Total build time is 24 hours including hardware assembly, firmware, ML model training, and software.

- Vehicle must be fully autonomous during the graded run (no manual joystick operation).

- Web server must be hosted locally — no external cloud services required for core functionality.

## 11.3 Out of Scope

- 3D printed or laser-cut custom chassis (use standard proto-board or purchased chassis).

- Multi-vehicle coordination or fleet management.

- Cloud-hosted backend or real-time database (e.g., Firebase) — optional if time permits.

- Voice control or gesture-based interaction.